Lab Experiment: 4

### Title: Playing Melody with Buzzer using Arduino

Course title: IOT Laboratory

Course code: CSE-460

4th Year 1st Semester Examination 2025

Date of Submission: 22/02/2026

Submitted to-

Dr. Md. Ezharul Islam, B.SC.(JU), M.ENGG.(JAPAN), PHD(JAPAN)

Professor

Department of Computer Science and Engineering

Jahangirnagar University

&

Samsun Nahar Khandakar

Assistant Professor

Department of Computer Science and Engineering

Jahangirnagar University

Savar, Dhaka-1342

| Class Roll | Exam Roll | Name |
| --- | --- | --- |
| 383 | 210903 | Abdullah Nazmus-Sakib |
| 379 | 210899 | Shah Sultan |
| 391 | 210911 | Md Khaled Mahmud |

### Lab–4: Playing Melody with Buzzer using Arduino

## 📘 1. Title Playing Melody with Buzzer using Arduino

## 🎯 2. Objective

- To interface a buzzer with an Arduino board

- To generate sound using digital signals

- To play a simple musical melody using programmed notes

- To understand tone generation using frequency and delay

## 🧰 3. Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno (or compatible board) | 1 |
| Buzzer (Active/Passive) | 1 |
| Jumper wires | As required |
| USB cable | 1 |
| Computer with Arduino IDE | 1 |

## 🔌 4. Circuit Connection

- Buzzer positive (+) terminal → Digital Pin 9 of Arduino

- Buzzer negative (–) terminal → GND of Arduino

This allows Arduino to send HIGH/LOW signals to produce sound waves.

## 💻 5. Program (Arduino Code)

int buzzerpin = 9;

int length = 15;

char notes[] = "ccggaagffeeddc ";

int beats[] = {1,1,1,1,1,1,2,1,1,1,1,1,1,2,4};

int tempo = 300;

void playTone(int tone, int duration) {

for (long i = 0; i < duration * 1000L; i += tone * 2) {

digitalWrite(buzzerpin, HIGH);

delayMicroseconds(tone);

digitalWrite(buzzerpin, LOW);

delayMicroseconds(tone);

}

}

void playNote(char note, int duration) {

char names[] = {'c','d','e','f','g','a','b','C'};

int tones[] = {1915,1700,1519,1432,1275,1136,1014,956};

for (int i = 0; i < 8; i++) {

if (names[i] == note) {

playTone(tones[i], duration);

}

}

}

void setup() {

pinMode(buzzerpin, OUTPUT);

}

void loop() {

for (int i = 0; i < length; i++) {

if (notes[i] == ' ') {

delay(beats[i] * tempo);

} else {

playNote(notes[i], beats[i] * tempo);

}

delay(tempo / 2);

}

}

## ⚙️ 6. Working Principle

- Arduino outputs square wave signals from Pin 9

- The buzzer converts electrical pulses into sound

- Different frequencies correspond to different musical notes

- Delay determines note duration (tempo)

- A sequence of notes forms a melody

## ▶️ 7. Procedure

- Collect all required components

- Connect buzzer to Arduino as shown

- Open Arduino IDE on the computer

- Write or paste the provided code

- Select correct Board and COM port

- Upload the program to Arduino

- Observe the buzzer playing the melody

## 📊 8. Result

The buzzer successfully produced a musical melody based on programmed notes, demonstrating sound generation using Arduino digital output.

## 🧠 9. Applications

- Alarm systems

- Doorbells

- Musical toys

- IoT notification devices

- Warning indicators in embedded systems

## ⚠️ 10. Precautions

- Ensure correct polarity of buzzer

- Avoid short circuits

- Use proper digital pin connection

- Check board and port before uploading

## 🏁 11. Conclusion

This experiment demonstrated how an Arduino microcontroller can generate sound and play melodies using a buzzer. It provided practical understanding of digital signal output, timing control, and basic sound synthesis — fundamental concepts in IoT and embedded systems.