Lab Experiment: 6

### Title: Interfacing LCD with Arduino to Display Temperature using LM35 Sensor

Course title: IOT Laboratory

Course code: CSE-460

4th Year 1st Semester Examination 2025

Date of Submission: 30/03/2026

Submitted to-

Dr. Md. Ezharul Islam, B.SC.(JU), M.ENGG.(JAPAN), PHD(JAPAN)

Professor

Department of Computer Science and Engineering

Jahangirnagar University

&

Samsun Nahar Khandakar

Assistant Professor

Department of Computer Science and Engineering

Jahangirnagar University

Savar, Dhaka-1342

| Class Roll | Exam Roll | Name |
| --- | --- | --- |
| 383 | 210903 | Abdullah Nazmus-Sakib |
| 379 | 210899 | Shah Sultan |
| 391 | 210911 | Md Khaled Mahmud |

## 📘 1. Title

Interfacing LCD with Arduino to Display Temperature using LM35 Sensor

## 🎯 2. Objective

- To interface a 16x2 LCD with Arduino

- To read temperature data from LM35 sensor

- To display real-time temperature on LCD

- To understand analog input and display output

## 🧰 3. Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| 16x2 LCD Display | 1 |
| LM35 Temperature Sensor | 1 |
| Potentiometer (1k–10k) | 1 |
| Jumper wires | As required |
| Breadboard | 1 |

## 🔌 4. Circuit Connection

### 🔹 LM35 Sensor Connection

- Pin 1 → 5V

- Pin 2 → A0 (Analog input)

- Pin 3 → GND

📌 As shown in the diagram (page 1), LM35 provides analog voltage output

### 🔹 LCD Connection

- RS → Pin 12

- E → Pin 11

- D4–D7 → Pins 5, 4, 3, 2

- VSS → GND

- VDD → 5V

- V0 → Potentiometer (contrast control)

📌 The full wiring layout is illustrated in the circuit diagram (page 2)

## 💻 5. Program (Arduino Code)

#include <LiquidCrystal.h>

int sensorpin = A0;

int temperature = 0;

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

void setup() {

pinMode(sensorpin, INPUT);

lcd.begin(16, 2);

lcd.print("Temperature:");

}

void loop() {

temperature = analogRead(sensorpin);

temperature = temperature * 0.48828125;

lcd.setCursor(0,1);

lcd.print(temperature);

lcd.print((char)223); // degree symbol

lcd.print("C");

}

## ⚙️ 6. Working Principle

- LM35 sensor outputs voltage proportional to temperature

- Arduino reads analog value (0–1023)

- Value is converted to temperature using formula:

Temperature (°C) = Analog Value × 0.48828125

- LCD displays temperature in real-time

- Potentiometer controls LCD contrast

## ▶️ 7. Procedure

- Collect all required components

- Connect LM35 sensor and LCD as per diagram

- Connect Arduino to computer

- Open Arduino IDE

- Write or paste the code

- Select correct board and port

- Upload the program

- Observe temperature displayed on LCD

## 📊 8. Observation

- LCD displayed “Temperature:” on first line

- Second line showed real-time temperature value

- Value changed when temperature around sensor changed

## 📈 9. Result

The temperature measured by LM35 sensor was successfully displayed on the LCD in real-time, demonstrating proper interfacing of sensor and display module with Arduino.

## 🧠 10. Applications

- Digital thermometers

- Weather monitoring systems

- Smart home automation

- Industrial temperature monitoring

- IoT display systems

## ⚠️ 11. Precautions

- Ensure correct LCD pin connections

- Adjust potentiometer for proper display visibility

- Avoid loose wiring

- Use correct power supply

- Do not overheat LM35 sensor

## 🏁 12. Conclusion

This experiment demonstrated how to interface both an input device (LM35 sensor) and an output device (LCD) with Arduino. It provided a clear understanding of analog data acquisition and real-time display, which are essential concepts in embedded systems and IoT applications.