Lab Experiment: 7

### Title: Sending Temperature Data from NodeMCU ESP8266 to ThingSpeak Cloud

Course title: IOT Laboratory

Course code: CSE-460

4th Year 1st Semester Examination 2025

Date of Submission: 12/04/2026

Submitted to-

Dr. Md. Ezharul Islam, B.SC.(JU), M.ENGG.(JAPAN), PHD(JAPAN)

Professor

Department of Computer Science and Engineering

Jahangirnagar University

&

Samsun Nahar Khandakar

Assistant Professor

Department of Computer Science and Engineering

Jahangirnagar University

Savar, Dhaka-1342

| Class Roll | Exam Roll | Name |
| --- | --- | --- |
| 383 | 210903 | Abdullah Nazmus-Sakib |
| 379 | 210899 | Shah Sultan |
| 391 | 210911 | Md Khaled Mahmud |

## 📘 1. Title

Sending Temperature Data from NodeMCU ESP8266 to ThingSpeak Cloud

## 🎯 2. Objective

- To interface LM35 temperature sensor with NodeMCU

- To connect NodeMCU to the internet via WiFi

- To send real-time temperature data to ThingSpeak cloud platform

- To visualize sensor data using an online dashboard

## 🧰 3. Components Required

| Component | Quantity |
| --- | --- |
| NodeMCU ESP8266 | 1 |
| LM35 Temperature Sensor | 1 |
| Micro USB Cable | 1 |
| Laptop with Arduino IDE | 1 |
| Internet Connection | 1 |
| ThingSpeak Account | 1 |

## 🔌 4. Circuit Connection

### 🔹 LM35 Sensor → NodeMCU

- Pin 1 → 3.3V

- Pin 2 → A0 (Analog Pin)

- Pin 3 → GND

📌 As described in your lab sheet (page 1), LM35 is connected directly to A0 for analog reading

## 💻 5. Program (Arduino Code)

#include <ESP8266WiFi.h>

#include <WiFiClient.h>

#include <ThingSpeak.h>

const char* ssid = "YOUR_WIFI";

const char* password = "YOUR_PASSWORD";

int val;

int pin = A0;

WiFiClient client;

unsigned long myChannelNumber = YOUR_CHANNEL_ID;

const char * myWriteAPIKey = "YOUR_API_KEY";

void setup() {

Serial.begin(9600);

delay(10);

WiFi.begin(ssid, password);

ThingSpeak.begin(client);

}

void loop() {

val = analogRead(pin) * 0.322265;

Serial.print("Temperature: ");

Serial.print(val);

Serial.println("*C");

ThingSpeak.writeField(myChannelNumber, 1, val, myWriteAPIKey);

delay(15000); // Required delay for ThingSpeak

}

## ⚙️ 6. Working Principle

- LM35 sensor outputs analog voltage proportional to temperature

- NodeMCU reads this analog value via A0 pin

- Value is converted into temperature (°C)

- NodeMCU connects to WiFi network

- Data is sent to ThingSpeak server using API key

- ThingSpeak stores and displays data as a graph

## ▶️ 7. Procedure

- Collect all components

- Connect NodeMCU to laptop via USB

- Install ESP8266 board package in Arduino IDE

- Select NodeMCU 1.0 (ESP-12E Module)

- Connect LM35 sensor properly

- Create ThingSpeak account and channel

- Get Channel ID and Write API Key

- Install ThingSpeak library

- Write and upload the program

- Open Serial Monitor to check readings

- Open ThingSpeak dashboard to view graph

## 📊 8. Observation

- Serial Monitor displayed temperature values

- Data was successfully uploaded to ThingSpeak

- Graph updated automatically in real-time

- Temperature values changed according to environment

## 📈 9. Result

The temperature data from LM35 sensor was successfully transmitted to the ThingSpeak cloud platform using NodeMCU ESP8266 and displayed in graphical form in real-time.

## 🧠 10. Applications

- Smart weather monitoring systems

- Remote temperature monitoring

- Industrial IoT systems

- Smart agriculture

- Home automation

## ⚠️ 11. Precautions

- Ensure correct WiFi credentials

- Use correct API key and Channel ID

- Maintain stable internet connection

- Add proper delay (≥15 seconds)

- Check wiring carefully

## 🏁 12. Conclusion

This experiment demonstrated the integration of sensors, microcontrollers, and cloud platforms in IoT systems. It provided practical knowledge of real-time data acquisition, wireless communication, and cloud-based monitoring, which are essential for modern IoT applications.